KiBoM PCB Bill of Materials

| Source File | D:\ 2018\ , \ , .sch |
| --- | --- |
| BoM Date | 12.09.2018 6:11:21 |
| Schematic Version |  |
| Schematic Date | 5/05/2018 |
| KiCad Version | Eeschema 4.0.7 |
| Component Groups | 71 |
| Component Count (per PCB) | 274 |
| Fitted Components (per PCB) | 274 |
| Number of PCBs | 1 |
| Total Component Count (for 1 PCBs) | 274 |

Component Groups

KiCad Fields (default)

Generated Fields

User Fields

Empty Fields

|  | Description | Part | References | Value | Footprint | Quantity Per PCB | Datasheet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Unpolarized capacitor | C | C68 | 0*p | C_0805_HandSoldering | 1 |  |
| 2 | Unpolarized capacitor | C | C1 C2 C3 C5 C6 C7 C8 C9 C70 C72 C73 C74 C75 C76 C77 C78 C79 C80 C81 C82 C83 C84 C85 C86 C87 C88 C89 C90 C91 C92 C93 C94 C95 C96 C97 C98 C99 C100 C101 c1 c2 c3 c5 | 0.1 | C_0805_HandSoldering | 43 |  |
| 3 | Unpolarized capacitor | C | C19 C20 C21 C22 C23 C24 C25 C26 C35 C36 C37 C49 C50 C51 C52 C53 C54 C55 C56 | 100p | C_0805_HandSoldering | 19 |  |
| 4 | Polarised capacitor | CP1 | C4 C29 | 100u | CP_Elec_5x4.5 | 2 |  |
| 5 | Unpolarized capacitor | C | C10 c4 | 10n | C_0805_HandSoldering | 2 |  |
| 6 | Unpolarized capacitor | C | C59 | 110p | C_0805_HandSoldering | 1 |  |
| 7 | Unpolarized capacitor | C | C34 | 120*p | C_0805_HandSoldering | 1 |  |
| 8 | Unpolarized capacitor | C | C40 | 12p | C_0805_HandSoldering | 1 |  |
| 9 | Unpolarized capacitor | C | C69 C71 | 15*p | C_0805_HandSoldering | 2 |  |
| 10 | Unpolarized capacitor | C | C33 | 156p | C_0805_HandSoldering | 1 |  |
| 11 | Unpolarized capacitor | C | C67 | 18p | C_0805_HandSoldering | 1 |  |
| 12 | Unpolarized capacitor | C | C13 C14 C15 C16 C17 C18 C43 C44 C45 C46 C47 C48 | 220p | C_0805_HandSoldering | 12 |  |
| 13 | Unpolarized capacitor | C | C32 | 240*p | C_0805_HandSoldering | 1 |  |
| 14 | Unpolarized capacitor | C | C66 | 30p | C_0805_HandSoldering | 1 |  |
| 15 | Unpolarized capacitor | C | C60 C63 | 33p | C_0805_HandSoldering | 2 |  |
| 16 | Unpolarized capacitor | C | C64 | 39p | C_0805_HandSoldering | 1 |  |
| 17 | Unpolarized capacitor | C | C11 C12 C31 C41 C42 | 470p | C_0805_HandSoldering | 5 |  |
| 18 | Unpolarized capacitor | C | C27 C28 C57 C58 C61 | 56p | C_0805_HandSoldering | 5 |  |
| 19 | Unpolarized capacitor | C | C38 C39 | 56р | C_0805_HandSoldering | 2 |  |
| 20 | Unpolarized capacitor | C | C65 | 75p | C_0805_HandSoldering | 1 |  |
| 21 | Unpolarized capacitor | C | C30 C62 | 91p | C_0805_HandSoldering | 2 |  |
| 22 | Diode | D | D0 | CDSOD323-T05C | D_SOD-323_HandSoldering | 1 | http://datasheet-pdf.com/PDF/CDSOD323-T05C-Datasheet-Bourns-916521 |
| 23 | Diode, alternativ symbol | D_ALT | D15 D16 D17 D18 D19 D20 D21 D22 D23 D24 | D | D_1206 MBR0530T1G | 10 | https://www.alldatasheet.com/datasheet-pdf/pdf/172204/ONSEMI/MBR0530T1G.html |
| 24 | LED generic | LED | D1 D2 D3 D4 D5 D6 D7 D8 D9 D10 D11 D12 D13 D14 | LED | LED_0805 | 14 |  |
| 25 | Diode, alternativ symbol | D_ALT | D25 | ZMD8.2v | D_2010 | 1 | http://www.alldatasheet.com/datasheet-pdf/pdf/81687/DIOTEC/ZMD8.2.html |
| 26 | LED generic | LED | D26 | on/of увч | LED_0805 | 1 |  |
| 27 | INDUCTOR | L | Dr1 Dr2 | 1R5 | L_1210 | 2 | http://datasheet-pdf.com/PDF/NLV32T-1R5J-PF-Datasheet-TDK-770663 |
| 28 | coaxial connector (BNC, SMA, SMB, SMC, Cinch/RCA, ...) | Conn_Coaxial | J4 | Dup (Pure-Signal) | SMA_THT_Jack_Straight | 1 |  |
| 29 | coaxial connector (BNC, SMA, SMB, SMC, Cinch/RCA, ...) | Conn_Coaxial | J6 | Input rx sdr | SMA_THT_Jack_Straight | 1 |  |
| 30 | Generic connector, single row, 01x03 | Conn_01x03 | J9 | Otput 1/Tx 2m/. 2/70sm/ | JST_EH_B03B-EH-A_03x2.50mm_Straight | 1 |  |
| 31 | coaxial connector (BNC, SMA, SMB, SMC, Cinch/RCA, ...) | Conn_Coaxial | J12 | Otput /rx-ant/ | SMA_THT_Jack_Straight | 1 |  |
| 32 | Generic connector, single row, 01x02 | Conn_01x02 | J2 | Otput Tx /1-50Mgz/ | JST_EH_B02B-EH-A_02x2.50mm_Straight | 1 |  |
| 33 | coaxial connector (BNC, SMA, SMB, SMC, Cinch/RCA, ...) | Conn_Coaxial | J10 | Otput XVTRs/2m/ | SMA_THT_Jack_Straight | 1 |  |
| 34 | coaxial connector (BNC, SMA, SMB, SMC, Cinch/RCA, ...) | Conn_Coaxial | J11 | Otput XVTRs/70sm/ | SMA_THT_Jack_Straight | 1 |  |
| 35 | Generic connector, single row, 01x03 | Conn_01x03 | J1 | input Tx 1/Germes/. 2/Odysej.Red-Pit.. | JST_EH_B03B-EH-A_03x2.50mm_Straight | 1 |  |
| 36 | Generic connector, single row, 01x04 | Conn_01x04 | J5 | input/Germes/Orion/ | JST_EH_B04B-EH-A_04x2.50mm_Straight | 1 |  |
| 37 | Generic connector, single row, 01x07 | Conn_01x07 | J3 | input/Odysej/Red-Pitay/hiqsdr | JST_EH_B07B-EH-A_07x2.50mm_Straight | 1 |  |
| 38 | Generic connector, single row, 01x08 | Conn_01x08 | J13 | k Lpf/6-160m/ | JST_EH_B08B-EH-A_08x2.50mm_Straight | 1 |  |
| 39 | Generic connector, single row, 01x02 | Conn_01x02 | J7 | on/of | GS2 | 1 |  |
| 40 | Generic connector, single row, 01x02 | Conn_01x02 | J15 | on/of 5v rele | GS2 | 1 | on/of 5v/12v/ rele |
| 41 | Generic connector, single row, 01x03 | Conn_01x03 | J14 | on/of 5v/12v/ rele | GS3 | 1 | on/of 5v/12v/ rele |
| 42 | Generic connector, single row, 01x04 | Conn_01x04 | J8 | power 12v+/- | JST_EH_B04B-EH-A_04x2.50mm_Straight | 1 | power 12v |
| 43 | IM Relay, standard version, monostable, switching current 2/5A, power 60W/62.5VA, voltage 220VDC/250VAC, THT package | IM06 | K1 K2 K3 K4 K5 K6 K7 K8 K9 K10 K11 K12 K13 K14 K15 K16 K17 K18 K19 K20 K21 K22 K23 K24 | IM06 | relay_gq-smd-a | 24 | http://www.te.com/commerce/DocumentDelivery/DDEController?Action=srchrtrv&DocNm=108-98001&DocType=SS&DocLang=EN |
| 44 | INDUCTOR | L | L30 L32 L40 | 100nH | L_1210 | 3 | NLV32T-R10J-PF |
| 45 | INDUCTOR | L | L20 | 120nH | L_1210 | 1 | NLV32T-R12J-PF |
| 46 | INDUCTOR | L | L10 L31 | 150nH | L_1210 | 2 | NLV32T-R15J-PF |
| 47 |  | INDUCTOR | L1 L2 L3 L4 L5 L6 L7 L8 L9 L11 L12 L13 L14 L15 L16 L17 L18 L19 L21 L22 L23 L24 L25 L26 L27 L28 L29 | T50 | L_Toroid_Vertical_L10.0mm_W5.0mm_P5.08mm | 27 |  |
| 48 | 50V Vds, 0.22 A Id, N-channel MOSFET, SOT-23 | 2N7002 | Q1 | 2N7002 | SOT-23 | 1 | https://www.alldatasheet.com/datasheet-pdf/pdf/15092/PHILIPS/2N7002.html |
| 49 | 45V Vce, 1A Ic, PNP Medium Power Transistor, SOT-89 | BCX53 | Q2 Q5 | BCX53 | SOT-89-3 | 2 | http://www.infineon.com/dgdl/bcx51_bcx52_bcx53.pdf |
| 50 |  | BFG135 | Q3 Q4 | BFG135 | SOT-223 | 2 |  |
| 51 | Resistor | R | R25 R26 R30 R31 | 10 | R_0805 | 4 |  |
| 52 | Resistor | R | R28 | 100 | R_0805 | 1 |  |
| 53 | Resistor | R | R1 | 10k | R_0805 | 1 |  |
| 54 | Resistor | R | R6 R7 R12 | 1k | R_0805 | 3 |  |
| 55 | Resistor | R | R24 | 2.2k | R_0805 | 1 |  |
| 56 | Resistor | R | R34 | 200 | R_0805 | 1 |  |
| 57 | Resistor | R | R27 R29 | 24 | R_0805 | 2 |  |
| 58 | Resistor | R | R2 R3 R4 R5 R10 R13 R14 R15 R16 R17 R18 R19 R20 R22 R35 R37 | 3k | R_0805 | 16 |  |
| 59 | Resistor | R | R21 | 50 | R_0805 | 1 |  |
| 60 | Resistor | R | R32 R33 | 51 | R_0805 | 2 |  |
| 61 | Resistor | R | R23 | 510 | R_0805 | 1 |  |
| 62 | Resistor | R | R11 R36 | 6.2k | R_0805 | 2 |  |
| 63 | Resistor | R | R8 R9 | R1k | R_0805 | 2 |  |
| 64 | 0.005-125MHz 1:1 RF Transformer, Unbalanced to Balanced Center Tap, CD542 | T1-4 | TR2 | MABAES0061 | t1-4 | 1 | https://www.digikey.com/product-detail/en/m-a-com-technology-solutions/MABAES0061/1465-1318-1-ND/4430015 |
| 65 | 0.005-125MHz 1:1 RF Transformer, Unbalanced to Balanced Center Tap, CD542 | T1-4 | TR1 | t1-4. MABAES0061 | t1-4 | 1 | https://www.digikey.com/product-detail/en/m-a-com-technology-solutions/MABAES0061/1465-1318-1-ND/4430015 |
| 66 | Positive 1.5A 35V Linear Regulator, Fixed Output 5V, TO-220/TO-263/TO-252 | L7805 | U7 | LM2940IMP-5.0 | SOT-223 | 1 | http://elcodis.com/parts/2220227/LM2940IMP-50.html |
| 67 | U1 | ULN2003 | U1 | ULN2003 | SOIC-16_3.9x9.9mm_Pitch1.27mm | 1 | http://datasheet-pdf.com/mobile/956153/ULN2003A.html |
| 68 | U5,U6 | ULN2004 | U5 U6 | ULN2004 | SOIC-16_3.9x9.9mm_Pitch1.27mm | 2 | http://datasheet-pdf.com/datasheet/Diodes/924207/ULN2004A.pdf.html |
| 69 | U4 | CD4011BM | U4 | cd4011bm | SOIC-14_3.9x8.7mm_Pitch1.27mm | 1 | https://www.belchip.by/sitedocs/00013462.pdf |
| 70 | U2,U3 | cd4028 | U2 U3 | cd4028 | SOIC-16_3.9x9.9mm_Pitch1.27mm | 2 | https://www.ti.com/lit/ds/symlink/cd4028b.pdf |
| 71 | INDUCTOR | L | blm1 blm2 blm3 blm4 blm5 blm6 blm7 blm8 blm9 blm10 blm11 blm12 blm13 blm14 blm15 blm16 | BLM21PG221SN1D | L_0805 | 16 | https://ru.aliexpress.com/item/100pcs-SMD-bead-FB-0805-220R-100MHZ-2012-221-220-ohm-25-2A-Ferrite-High-frequency/32816002855.html?spm=a2g0s.9042311.0.0.274233edcT5Tse |